|  | Internet of Things Practical #1 |  |  |
| --- | --- | --- | --- |
| Name | SIMRAN SINGH | Division | D |
| Class | T.Y.B.Sc.DS | Roll Number | 23302D0004 |
| Practical 1 | Starting Raspbian OS, familiarizing with raspberry pi components and Interface, connecting to ethernet, monitor, USB. |  |  |
|  |  |  | Date: 1-7-25 |
| Procedure to install Rasbian OS |  |  |  |
| 1. Prerequisites Before installing Raspberry Pi OS, ensure you have: - A Raspberry Pi board (any model) - A microSD card (minimum 8GB, Class 10 recommended) - A microSD card reader (if your computer doesn’t have one) - A computer (Windows/macOS/Linux) - A stable internet connection (for downloading the OS image) - Raspberry Pi Imager (official tool) or Balena Etcher (alternative) 2. Download Raspberry Pi OS - Visit the official Raspberry Pi website: https://www.raspberrypi.com/software/ - Under "Raspberry Pi OS", choose: - Raspberry Pi OS (32-bit) – Recommended for most users - Raspberry Pi OS Lite (32-bit) – No desktop (for headless setups) - Raspberry Pi OS (64-bit) – For advanced users (better performance on Pi 3/4/5) - Download the .zip or .xz file. 3. Flash Raspberry Pi OS to the microSD Card Method 1: Using Raspberry Pi Imager (Easiest) - Download Raspberry Pi Imager from https://www.raspberrypi.com/software/ - Insert the microSD card into your computer. - Open Raspberry Pi Imager. - Choose OS → Select Raspberry Pi OS (32-bit or 64-bit). - Choose Storage → Select your microSD card. - (Optional) Click the gear icon (⚙️) to: - Set hostname (e.g., raspberrypi.local) - Enable SSH (for remote access) - Configure Wi-Fi (if using headless mode) - Set username & password (default: pi/raspberry) - Click "Write" and wait for the process to complete. 4. Boot Raspberry Pi for the First Time - Insert the microSD card into the Raspberry Pi. - Connect peripherals (keyboard, mouse, monitor via HDMI). - Power on the Pi using a USB-C (Pi 4/5) or Micro-USB (Pi 3/Zero) power supply. - First-time setup (if using Raspberry Pi OS with Desktop): - Select language, timezone, and keyboard layout. - Set a new password (default: pi/raspberry). - Connect to Wi-Fi (if needed). - Update software (recommended). |  |  |  |
| Configuring Raspi. |  |  |  |
| 1. Initial Setup - Run sudo raspi-config - Set hostname (iot-pi-1) - Change default password - Set timezone & keyboard 2. Network - Assign static IP (edit /etc/dhcpcd.conf) - Enable WiFi (nmcli or raspi-config) 3. Remote Access - Enable SSH (via raspi-config or touch /boot/ssh) - Enable VNC (for GUI access) 4. IoT Hardware Setup - Enable I2C, SPI, Serial in raspi-config - Add user to GPIO group: 5. Security - Update system: 6. Test - Reboot & verify settings: |  |  |  |
| State the Linux Commands used for File System. |  |  |  |
| 1. Navigation - pwd – Show current directory - ls – List files/folders - ls -l (detailed) - ls -a (show hidden) - cd <dir> – Change directory - cd .. (go back) - cd ~ (go home) 2. File Operations - touch <file> – Create empty file - cat <file> – Display file content - nano <file> – Edit file (Ctrl+X to exit) - cp <source> <dest> – Copy file/folder - mv <source> <dest> – Move/rename - rm <file> – Delete file - rm -r <dir> (delete folder) - rm -f (force delete) 3. Directory Operations - mkdir <dir> – Create folder - rmdir <dir> – Delete empty folder - tree – Display folder structure 4. Permissions - chmod +x <file> – Make executable - chown <user>:<group> <file> – Change owner 5. Disk & Storage - df -h – Show disk space - du -sh <dir> – Check folder size - mount – List mounted drives 6. Search & Find - find / -name "file.txt" – Search system - grep "text" <file> – Find text in file 7. Compression - tar -czvf archive.tar.gz <dir> – Create .tar.gz - unzip file.zip – Extract ZIP |  |  |  |
| State the Linux Commands used for Networking |  |  |  |
| 1. Basic Network Info - ifconfig or ip a – Show IP addresses - iwconfig – Wireless network info - hostname -I – Quick IP check (RPi) - ping google.com – Test internet connection 2. Connection Management - nmcli – NetworkManager CLI tool - nmcli dev wifi list – Scan WiFi - nmcli dev wifi connect "SSID" password "PASS" – Connect to WiFi - iwlist wlan0 scan – Detailed WiFi scan 3. SSH & Remote Access - ssh user@ip – Connect via SSH - scp file.txt user@ip:/path – Copy files over SSH - rsync -avz /local/path user@ip:/remote/path – Sync files 4. Port & Firewall - netstat -tuln – List open ports - ss -tuln (modern alternative) - sudo ufw allow 22 – Allow SSH (UFW firewall) 5. DNS & Hosts - dig google.com – DNS lookup - nslookup google.com – Check DNS resolution - Edit /etc/hosts – Manual hostname mapping 6. Troubleshooting - traceroute google.com – Network path check - mtr google.com – Advanced traceroute - arp -a – Show ARP table (local devices) |  |  |  |